8QUEENS

"Join us for a fun and challenging activity at our upcoming fest! Can you conquer the 8 Queen Puzzle and earn the title of chessboard champion? Come show off your strategic skills and see if you have what it takes to solve this classic puzzle."

Objective: On Chess board the player has to place 8 queens in such a way that no two queens threaten each other ; thus a solution requires that no two queens share the same row , column or diagonal.

Components required:

- Hardware:

1.Arduinouno

2 Push button (64)

3.Connecting wires

4.Jumper wires

5.Led-Red(1)

-Green(1)

6.Battery

- Software:

1.Arduino ide

Circuit Diagram:

Software code:

1.push button check code

const int ROWS = 4;

const int COLS = 4;

const int rowPins[ROWS] = {2, 3, 4, 5};

const int colPins[COLS] = {6, 7, 8, 9};

// Define a struct to hold the row and column indices of a button press

struct point {

int row;

int col;

};

// Create an array of ButtonPress structures to hold the coordinates

point p[8];

void setup() {

Serial.begin(9600);

for (int r = 0; r < ROWS; r++) {

pinMode(rowPins[r], OUTPUT);

digitalWrite(rowPins[r], HIGH);

}

for (int c = 0; c < COLS; c++) {

pinMode(colPins[c], INPUT_PULLUP);

}

}

void loop() {

int count = 0;

bool buttonPressed = false;

while (!buttonPressed) {

for (int r = 0; r < ROWS; r++) {

digitalWrite(rowPins[r], LOW);

for (int c = 0; c < COLS; c++) {

if (digitalRead(colPins[c]) == LOW) {

// Store the button press coordinates in the array of structures

p[count].row = r;

p[count].col = c;

count++;

while (digitalRead(colPins[c]) == LOW) {}  // Wait for key release

if (count == 8) {  // Stop accepting input after 8 button presses

buttonPressed = true;

break;

}

}

}

digitalWrite(rowPins[r], HIGH);

if (buttonPressed) {

break;

}

}

}

// Print out the button press coordinates

for (int i = 0; i < count; i++) {

Serial.print("Button pressed at: (");

Serial.print(p[i].row + 1);  // Add 1 to row and col to convert from zero-indexed to one-indexed

Serial.print(", ");

Serial.print(p[i].col + 1);

Serial.println(")");

}

}

2.main code

#include <stdint.h>

const int ROWS = 4;

const int COLS = 4;

const int rowPins[ROWS] = {2, 3, 4, 5};

const int colPins[COLS] = {A0, A1, A2, A3};

// Define a struct to hold the row and column indices of a button press

struct point {

int row;

int col;

};

//check for the cooridinate of the queen placed

bool issafe(int arr[4][4], int x, int y) {

//check for row

for (int i=0;i<4;i++) {

if (arr[x][i]==1 && i!=y) { // check for row and exclude current position

return false;

}

if (arr[i][y]==1 && i!=x) { // check for column and exclude current position

return false;

}

}

//check for left diagonal

int row=x;

int col=y;

while(row>=0 && col>=0) {

if (arr[row][col]==1 && (row!=x || col!=y)) { // check for left diagonal and exclude current position

return false;

}

row--;

col--;

}

//check for right diagonal

row = x;

col = y;

while (row >= 0 && col < 4) {

if (arr[row][col] == 1 && (row!=x || col!=y)) { // check for right diagonal and exclude current position

return false;

}

row--;

col++;

}

return true;

}

// Define the game function

bool game(int arr[4][4],struct point p[4]) {

for (int i=0; i<4; i++) {

if (!issafe(arr, p[i].row, p[i].col)) {

return false;

}

}

return true;

}

void setup() {

Serial.begin(9600);

for (int r = 0; r < ROWS; r++) {

pinMode(rowPins[r], OUTPUT);

digitalWrite(rowPins[r], HIGH);

}

for (int c = 0; c < COLS; c++) {

pinMode(colPins[c], INPUT_PULLUP);

}

}

void loop() {

for(int i=0;i<10;i++){

// Create an array of ButtonPress structures to hold the coordinates

struct point p[4];

int s[4][4]={0};

int count = 0;

bool buttonPressed = false;

while (!buttonPressed) {

for (int r = 0; r < ROWS; r++) {

digitalWrite(rowPins[r], LOW);

for (int c = 0; c < COLS; c++) {

if (digitalRead(colPins[c]) == LOW) {

// Check if the button has already been pressed

bool alreadyPressed = false;

for (int i = 0; i < count; i++) {

if (p[i].row == r && p[i].col == c) {

alreadyPressed = true;

break;

}

}

if (!alreadyPressed) {

// Store the button press coordinates in the array of structures

p[count].row = r;

p[count].col = c;

count++;

}

while (digitalRead(colPins[c]) == LOW) {}  // Wait for key release

if (count == 4) {  // Stop accepting input after 8 button presses

buttonPressed = true;

break;

}

}

}

digitalWrite(rowPins[r], HIGH);

if (buttonPressed) {

break;

}

}

}

// Print out the button press coordinates

for (int i = 0; i < count; i++) {

Serial.print("Button pressed at: (");

Serial.print(p[i].row );  // Add 1 to row and col to convert from zero-indexed to one-indexed

Serial.print(", ");

Serial.print(p[i].col);

Serial.println(")");

}

for(int i=0;i<count;i++){

s[p[i].row][p[i].col]=1;

}

if (game(s,p)) {

pinMode(12,OUTPUT);

digitalWrite(12,HIGH);

Serial.println("You won!");

delay(10000);

digitalWrite(12,LOW);

}

else {

pinMode(13,OUTPUT);

digitalWrite(13,HIGH);

Serial.println("You lost!");

delay(10000);

digitalWrite(13,LOW);

}

delay(1000);

}

}

OUTPUTS: